INTRODUCTION to the software development.

I do not pretend to be either a scientist or a computer expert but I have achieved good results in corrosion control field work and have used computers over the past 15 years.

During this time I have developed some unique concepts which are successful in cathodic protection field work but which have not been generally understood.  In order to test the credibility of my theories I have made models of the various corrosion and cathodic protection functions on computer spread sheets.  These models reflect the results which are obtained in practical field work and in physical models which I have constructed over the years from electronic components, sand trays etc.

I am attempting to combine all of the features, which I have created, into a single spread sheet which will show a complete geographical area with all the cathodic protection systems integrated.   Each installation has an effect on all of the others to the extent that I have experienced adjustments at one location altering readings obtained hundreds of miles away.

For example, an oil and gas production operation will consist of groups of wells feeding into a 'spider' of  small steel pipelines feeding into a network of 'delivery lines' feeding into manifolds on the main trunk pipelines that supply the tank farms.  DC electrical current flows through this network using the earth as its return path, and a typical operation will have 30 or 40 cathodic protection installations.

The function of these installations is to impress electrical charges into the earth and drain it from the pipelines so that all the metal is below its 'corrosion potential'.  Cathodic protection is very successful and cost effective but there continue to be pipeline failures through corrosion that was thought to be controlled.

The us of a computer might help in two ways.

1.	The computer makes it possible to confirm the credibility of electrical 	measurements.

2.	The computer can display an overview of the whole system incorporating all 	available data.

At present there is no internationally agreed criteria for cathodic protection due to difficulties in making credible field measurements.   In addition, it is very difficult to visualise the 'interference' effects of one installation on another, as there have been literally millions of readings recorded concerning most cathodic protection systems.

It is hoped that a computer model of a complete area of pipelines would help to eliminate the risk of failure and clarify the science of cathodic protection in general.  This would be of considerable benefit as most of the worlds energy supply is transported through steel pipelines which sometimes fail with disastrous results.