ACT-DYNAMIC

CORROSION CONTROL SERVICE

1 Julius Hill, Off Harvest Ride, Warfield, Bracknell, Berks, RG12 6UN.

Tel/fax 0344 57105

THE ALEXANDER CELL

ABSTRACT

The Alexander Cell is a device which monitor the corrosion control system which is applied to the external surface to buried pipelines.   It was invented over ten years ago and proved successful in field use, but the need for it has only recently become recognised on an international scale with the unexpected failures which are occurring to pipelines world-wide.  The manufacture and marketing of this simple device is to be re-launched and offers a business opportunity to the right investor.

INTRODUCTION

The inventor was responsible for the corrosion control of  an oil producing company and was faced with a problem which contemporary technology had failed to resolve.   The recommended measuring techniques did not render credible results and field experimentation revealed a major error in the fundamental logic.  He developed an arrangement of metal pieces which represented the configuration of a natural corrosion cell which is found on buried pipelines and which could be subjected to the same environment as the pipeline metal and the corrosion control system.  This could then be used by applying the same principal as installing a 'corrosion coupon', which is the accepted method of long term monitoring in general use in many industries and science.

The Alexander Cell is a short term method of monitoring the corrosion control based on  Farradays law, that the amount of metal going into solution is directly proportional to the DC electrical current discharged in the corrosion reaction.   The Alexander Cell creates an environment in which the reaction current can be measured in closed circuit and this has not been achieved by any other method of monitoring.

THE ARRANGEMENT

After consultations with top scientific institutions and authorities the Alexander Cell

is a steel anode and cathode set in a block of plastic the base of which consists of steel electrodes.

PROCEDURE TO USE THE CELL

The anode and the base electrodes are cleaned to bright metal and the cell is placed on the earth above the pipeline.   A sample of the earth is placed on top of the cell bridging the anode and the cathode and the status of the meter is noted.

The cell is then connected to the pipeline metal via the cathodic protection test facility, and the reading noted again.

INTERPRETATION

The first reading appears when the earth sample bridges the anode and cathode and the reaction discharges the corrosion current through the circuit.  The change in reading occurs when the cell is connected to the pipeline and reflects the effect of the corrosion control on the actual corrosion reaction in the cell.   If corrosion is halted the current will stop and there is sufficient shift in the pipeline to earth equilibrium for cathodic protection.

ADVANTAGES TO THE USER

There is no internationally agreed criteria for cathodic protection at present and the Alexander Cell provides a simple and inexpensive method whereby a pipeline operator can ensure that corrosion has been halted.

A simple survey can detect any areas where protection is not complete and adjustments can be made to cover these areas.  Remedial work can prevent the enormous financial and ecological risk of a pipeline failure due to external corrosion.

Potential users include major oil and gas production and distribution organisations throughout the world who have hundreds of thousands of buried pipeline and who are aware of the consequences of pipeline failure.  The majority of these operators are seeking a credible condition monitoring system as their assets age.  Cathodic protection has proved remarkably cost effective in prolonging the useful life but leaks are beginning to occur and there is no other short term measuring method which is as simple and successful as the Alexander Cell.

OUR NEEDS

ACT-DYNAMIC (UK) Limited  needs funds and personnel to manufacture and market this product.

There is no further development required for the general purpose model of the Alexander Cell, which is designed for use over pipelines buried on land, but operators have expressed an interest in developing the system for application to offshore pipelines.  A notional design has been drawn for this purpose but its full development and field trials would required considerable funding and management in co-operation with at least one prospective client.

There is at present no satisfactory method of monitoring the performance of cathodic protection on offshore pipelines.

We require assistance and advice on pricing and marketing of this product.