THE COMPLEXITY OF THE MODEL

A typical application for such a computer model would be an oil and gas production operation of a major international company.  This would consist of a nationwide network of pipelines transporting the product from source 'location' to the export facilities.

µ §

Each location consists of a "spider" of welded steel pipelines, bringing

oil and gas from wells, which are spread over several square

kilometers, to gathering centres, known as 'flowstations'.  Some wells are connected to injection pipelines carrying gas

or chemical inhibitors from the flowstation to be injected

downhole. These pipelines can be over a kilometer long and

some are coated, buried and connected to the CP system, while

others are bare steel with ground contact, or earthed through

their steel supports.

At the flowstation itself, all metal is earthed to a common

ground potential, for spark prevention reasons. This earthing

system consists of a bare copper ring main which runs round  the periphery of the station enclosure.  All buried pipework is coated, but pipe supports and stantion  bases are frequently in direct contact with the ground.  The result is a very complex area of DC electrical flux which is difficult to visualise and time consuming to analyse

A delivery pipeline will carry the product to a manifold on the main trunk pipeline which in turn carries it to a tank farm.

'Impressed current' cathodic protection installations each protect many miles of pipelines. In some cases this is a section of a major trunk line and in some cases many gathering lines and delivery lines.  Some installations work independantly and some are linked through direct connection through the same pipe length.  In some cases the pipes cross each other underground and in some cases electrical bonds are made through variable resistors, in an attempt to 'ration' the current.

At some locations 'sacrificial' anodes have been attached to the pipes, tank bottoms etc, and these cause a cirtain amount of confusion to the analysis.

The whole pipeline network can be imagined as an electrical circuit and the problem is to define the paths of all currents and the balance of all potentials in that circuit.   Current is impressed into the system  at  'groundbeds' where transformer-rectifiers increase the potential of the ground, causing a current to flow onto the pipelines.  The negative terminals of the transformer-rectifiers are connected to the pipelines, thus draining the current from them.

AVAILABLE DATA

Voltage measurements known as  'pipe-to-soil potentials' have been recorded with respect to most pipelines as well as the amount of current from these pipelines to the negative terminal of the cathodic protection transformer rectifier.

We therefore have a DC electrical circuit and a certain amount of recorded data and can build a computer model which reacts to the input of the survey results.  Using Kirchovs and Ohms laws, the computer can be programmed to display the points at which current is leaving the metal and entering earth.   These are the sites where corrosion is occuring.

Such analysis has been carried out manually in the past, but requires the engineer to be familiar with the complete network and make many 'informed guesses'.

PROGRESS SO FAR

A bi-annual CP survey has been conducted with the data reported directly into a computer by the contractors survey team.   DEVELOPMENT

The program is being developed while "in use" and, at present, is limited to disclosing cathodic protection defects which could have been detected by any competant engineer.

However, historical data, from old records, has been subject to this program which has already revealed the causes of leaks that have actually occurred.

It is realised that human hindsight can normally perform this function but, by highlighting all the CP system defects as soon as the data is gathered, this tool would allow the

engineer to decide his priorities more clearly, and in many instances, would have prevented the leaks.

It is clear that the real cause of some of these particular leaks, was the huge amount of infomation presented to the engineer, which could not be examined and analysed manually.

WHAT IS MANUAL ANALYSIS?

It is normal practice to examine survey results page by page, looking for individual readings which are below a particular criterion.  The next step is to examine related readings

which are on the same page, and then to turn from one page to another, examining related readings. The weight of information relating to the leak area, obviously, clouded the

issue before the application of the computer analysis.

CP is so complex that it is difficult to follow any line of logic and even more difficult to visualise the knock-on effects of all the readings that are available.

HOW DOES THE COMPUTER HELP?

The program under development  has a display which highlights the electrical nodes where the current readings do not, apparently, balance according to the laws of DC electricity

(Kirchov).  It is clear that this condition cannot exist, and

that the measurements must be inaccurate or the circuit diagram incorrect.

A sub-routine is being developed to explore the possibility that polarities have not been noted correctly.  This can cope with any node with up to six connected conductors, at the

present stage of development. This will define the

arrangement of polarities that will give a balanced result and could identify any pipeline that is discharging currentfrom the system.

PIPE-TO-SOIL POTENTIALS

The computer analysis of pipe-to-soil potential readings is

being approached on a statistical basis, as there is no

international agreement with respect to a definative criterion, at present.

The computer is programmed to display certain conditions graphically, where they suggest that the electrical pressures might cause current to leave the pipelines.  Once more, no

contentious theories are being utilised, but the massive

number of voltage readings that are already available, are being displayed on a simple graphical format which allows more of the relevant data to be examined at one instance.

TESTING A HYPOTHESIS

One notion, that has been applied to historical data, has proved to be accurate.  This uses potential shifts, calculated from voltage readings between unconnected steel

and other steel which is connected to the negative side of the CP system.  Both measurements are normally made, in practice, with reference to an electrode in the same position at each location.

It has been noticed that there is a tendency for corroded pipelines to have fallen into an identifiable group, which is being used as an indicator to identify pipelines which

require special attention.

In this case, the supporting theory is too vague to report to this conference, but the results so far, look very promising. The computer is being used to perform boring, repetitive

calculations that would be time consuming and may not be of consequence. However, now that this routine is in place, the computer can test the theory against a limitless amount of

data at no extra cost.

It is easy to compare the theoretical current flow, deduced from the voltage readings, with the actual current readings,taken in the field, as these are on the same spread sheet. The program simply brings these together in graphical display, on the same screen.

FUTURE POSSIBILITIES.

Future development of this project is to design and implement the integration of all the oil fields and trunk line cathodic protection systems so that one gigantic spread-sheet can

handle every voltage and current reading taken in the region.

This will be a relatively simple achievement, as spreadsheets can now handle enormous amounts of data.

The more difficult task will be to make it possible to compare data historically, by linking all the spreadsheets containing every reading ever taken so that every reading can be related to every other reading, both topographically and chronologically.

FEASIBILITY

A feasibility study has been carried out in the UK, confirming that it is possible to construct a computern software product  which will display a complete map of all pipelines etc. in the whole of an operation region.  It will be possible to zoom in on any detail and view this in three

dimensions.   The computer will be capable of displaying the

calculated amount of CP current and direction of flow at any node of the whole circuit.

This display would provide user friendly access to a data base containing tabular or graphical information, relating to the location selected on the map.  This data-base could

contain all data relating to the asset depicted on the display.  When this stage of the development has been reached as a cathodic protection tool, then it would seem opportune

to extend the data base to all other information.

USER FRIENDLY GRAPHICS

It is visualised that potentials will be represented by shading and colours and current flow by arrows such as are used on television weather reports. This would allow for product flow and pressure to be represented in the same way and it would enable this type of computer handling to be integrated with a total condition monitoring and operation control system.

Such a program would take time and expertise to construct, but would be a model on which all future development could be based.   It could even contain all the drawings etc. (in CAD) which are presently held in drawing office archives.

The feasibility of such a program may seem far fetched, but there is already a transport planning program in the UK that is based on a similar concept.  It consists of a map of most

of the roads in the UK through which the user can access a

wealth of information relating to any location.